PROBLEM SET NUMBER 1

Due Monday, Ocober 15, 2007 (in lecture)

# Reading Assignment for week of October 8

Read the Course Information and Policy and Chapra & Canale Chapter on “Approximations and Round-Off Errors”

# Problem Set # 1

This Problem Set is to be done by your team.  Teams will be assigned This week of October 8.  Reminder:  Unless otherwise specified, it is always permissible (but not required) to employ a spreadsheet, MATLAB, or some other computer program for the “number crunching” portions of Problem Sets.

Learning Objectives for Week of October 8

- Learn concepts of machine representation of real numbers.

- Compute and use true and approximate errors.

- Observe how the machine representation of numbers affects the precision of computations.

- Learn application of numerical differentiation as derived from Taylor Series.

Assignment

Suppose, for questions 1 through 4, that you are using a computer with a floating point system having a 5-digit mantissa and exponents of ±0 through ±99 (Base 10).

- If the computer hardware normalizes after every operation, then the smallest non-zero positive number is 0.10000E-99.  Numbers can be either negative or positive.

- With normalization, what is the machine representation of the difference: 		0.14358E-99 – 0.14359E-99

- If the hardware does not normalize, what is the smallest non-zero positive number that can be represented?

- What is the smallest possible number (algebraic)?

- If the hardware normalizes, how many different floating-point numbers can the hardware represent (big and small, positive and negative)?

- What is the smallest positive floating-point number that can be added to one and, after chopping, still yield a number larger than one?  This is machine epsilon.

- In different ranges (magnitudes) the spacing between representable numbers is different.  What is the absolute difference between 0.10000E+03 (or 100) and the next largest and smallest numbers available on this machine? Express the result in normalized form.

- Evaluate the function:     at x = 1.20 and 1.22 twice, once using full precision on your calculator or computer and once using 4-digit arithmetic.  Evaluate the error and suggest a reason for the difference in the error.